Practical notes

Equipment: (per group)

1 ammeter

1 voltmeter or multimeter

2 bulbs (3.5V)

2 cells or 1 power pack set to 3V

Leads

Demonstrate equipment to class. Explain how to use multimeter on 20V DC range – bottom and middle holes

Draw the circuit in books. Measure current in circuit and and voltage across the bulb. Record them

Add a second bulb in series. Measure the current and the voltages across each bulb and across the power pack.

Questions – answer in books in full sentences

1/	How does the current change when a second bulb is added? What happened to the brightness of the bulbs? Explain why (use the word ‘resistance’ in your answer)

2/	Look at the three voltages measured in circuit 2. Is there a link between the voltages across the two bulbs and the voltage across the power pack? (Hint – try adding the two bulb voltages together).

3/	Does an ammeter have a big or small resistance? How is it always connected in a circuit?

4/	Does a voltmeter have a big or small resistance? How is it connected in a circuit?

5/	Does an ammeter take the same current as the bulb?

6/	Does the voltmeter take the same current as the bulb?