Investigation planning sheet – current voltage graph of a component

Investigate how the resistance of a component changes as the current through it changes

Think about :

- How can you use the variable resistor in this circuit to change the voltage and current?

- How many different values of voltage and current will you need?

- How could you change the above circuit to investigate a bulb or a diode?

- For each set of readings :

-	What measurements will you make?

-	How many readings will you take?

-	How will you record your results?

-	How will you make sure you do not damage the component?

- Ask your teacher to check your circuit before
you begin the investigation.

- For each set of results, plot a graph of current (y axis) against voltage (x axis).

- Explain the conclusions you draw from your
results.

Are the graphs the same shape?

What happens to the resistance of each component as the voltage is increased?

| You might choose to use some of these : |  |  |
| --- | --- | --- |
|  |  | ammeter |
|  |  | voltmeter |
|  |  | low-voltage power supply |
|  |  | variable resistor |
|  |  | connecting leads, crocodile clips |
|  |  | wires of nichrome or constantan to make the resistor |
|  |  | (all of the same diameter, eg. 0.31 mm) |
|  |  | Bulb |
|  |  | Diode or LED |

potential difference (in V)

current (in A)

Resistance  =

(in )