Revision

QUIZ

31   Circuits

Use these questions to help you revise (cover the right-hand side with a sheet of paper).

| 1 | What is an electric current? |  | ... | a flow of electrons (–), moving from the negative terminal to the positive terminal. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Where in a circuit should you put an ammeter? |  | ... | in series with the component whose current you want to measure. |  |  |
| 3 | Where in a circuit should you put a voltmeter? |  | ... | in parallel, across the component whose p.d. you want to measure. |  |  |
| 4 | Draw a circuit diagram of a cell, switch, lamp, ammeter and variable resistor in series. |  | ... |  |  |  |
| 5 | When a p.d. of 10 V is applied across a resistor, 2 A flows. What is the resistance? |  | ... | resistance, |  | R =p.d., Vcurrent, I =10 V2 A=5 ohms (5Ω) |
| 6 | A resistance of 4  is carrying a current of 2 A, and is in series with a resistor of 6 . What is the total resistance and what is the current through the 6  resistor? |  | ... | total resistance = 4  + 6  = 10  current through each resistor (in series) = 2 A |  |  |
| 7 | A resistor of 2  is in parallel with a resistor |  | ... | the smaller resistor (2 ) has the bigger |  |  |
|  | of 4 . Which resistor has the bigger current? |  |  | current. (It will be twice as big.) |  |  |
| 8 | In the diagram, each cell has a p.d. of 1.5 V. What is the total p.d.? |  | ... | 6 V |  |  |
| 9 | What does the slope of a current–voltage graph tell you? |  | ... | the steeper the graph, the less the resistance. |  |  |
| 10 | Why is the current–voltage graph for a filament lamp a curved line? (p. 259) |  | ... | as more current flows, the metal filament gets hotter, so its resistance increases, so the graph gets flatter. |  |  |
| 11 | What do the letters MCB and RCD stand for? |  | … | Minature circuit breaker and Residual Current device |  |  |
| 12 | What advantage does a circuit breaker have over a fuse? |  |  | Circuit breakers can be reset, whereas fuses have to be replaced. |  |  |