U Values

| Compare the U values for the parts of a building shown in the table. Part of building | U-Value |
| --- | --- |
| 270mm Cavity wall without insulation | 1.0 |
| 270mm Cavity wall, with insulation | 0.6 |
| Single-glazed window | 5.0 |
| Double-glazed window | 2.9 |
| Roof material with 50mm insulation | 0.6 |
| Roof material with 100mm insulation | 0.3 |

1. How many times more heat is lost through a single glazed window compared to a double glazed window?

2. How many times more heat is lost through a cavity wall without insulation than a cavity wall with insulation?

2. Fill in this table for the classroom you are in now.  You will only need to consider the external walls.

| Part of Building | Length m (estimate) | Height m (estimate) | Area m2 | Energy lost per second (watts) |
| --- | --- | --- | --- | --- |

Assume that the outside temperature is 10oC and the inside temperature is 20oC. You will also need to decide which U value best fits the room you are in.

3. Use the relationship Energy = Power x Time to calculate how much energy would be lost from each of the above in one hour.